| BMW Specifications for exhaust test |  |
| --- | --- |
| ### Data record: 120 (ECE); ### Date of issue: 11/2017. |  |

## 1. Vehicle data

### E series:; ### I01. ### Model:; ### i3 94 (+REX). ### Vehicle version:; ### saloon. ### Engine model:; ### IB1P25B. ### Registration number:; ### IB1. ### Displacement:; ### 647 cc. ### Engine power:; ### 55 kW / 4300 rpm. ### EC homologation number:; ### e1*2007/46*1213*--. ### EU emission approval number:; ### EG 715/2007*2015/45ZA. ### Europ. exhaust emission stage and code:; ### Euro 6 W. ### Special technical features:; ### Range Extender. ### Status:; ### 03/2016. ### manufacturer's key number:; ### 0005. ### BMW type code number:; ### 1Z81..

## 2a. OBD method

### OBD method:; ### OBD signal readout: MIL status + readiness codes + P-codes. ### Note:; ### Petrol engines from exhaust emission stage EURO 6 Code W: At least 5 readiness codes must be set (value=0)' Readiness codes KATALYSATOR (CATALYSATOR) and SEKUNDÄRLUFTSYSTEM (SECONDARY AIR SYSTEM) may have value '1', i.e., not checked yet, as these require 'long time needed' test procedures...

## 2b. Nominal values (SI engine w. cat. converter)

### Shifting:; ### Automatic gearshift. ### Catalytic converter conditioning:. ### Engine temperature:; ### 70 °C / alternatively: oil temperature 60 °C. ### Idle speed:; ### 2000 rpm - 2500 rpm. ### Part-load engine speed:; ### 2600 rpm - 3000 rpm. ### CO content, idle speed:; ### 0.3 % v/v. ### CO content, part load:; ### 0.2 % v/v. ### Lambda value:; ### 0.97-1.03. ### Probe type:; ### Broad-band probe. ### Current min.:; ### -0.115 mA. ### Current max.:; ### +0.077 mA. ### Voltage increase min.:. ### Voltage min.:. ### Voltage max.:. ### Exhaust extraction point:; ### Measurement at tailpipe. ### Number of exhaust systems:; ### 1. ### HC content, idle speed*:; ### 200 ppm. ### HC content, part load*:; ### 100 ppm. ### Note:; ### HC values do not apply to exhaust inspections in Germany and in countries of the European Community. ### More detailed information can be found in the BMW Service Information Testing instruction for exhaust-gas test on BMW vehicles with alternative drives. Exhaust-gas test mode for forced running of the combustion engine: - Raise bonnet. - Establish operating readiness by pressing the brake pedal and operating the start/stop button at the same time. - The display of the instrument cluster (KOMBI) is set to "Ready". - Move selector lever to "P" position. - Operate accelerator pedal: Combustion engine starts and can be kept idle or in increased idle position depending on the accelerator pedal position. - End: Press the start/stop button...

## 3. Engine-speed pickoff

### Engine-speed pickoff:; ### 16-pin OBD socket. ### Installation location:; ### At the bottom of left A-pillar. ### Type of OBD:; ### EOBD..

## 4. Visual inspections

### Exhaust gas-relevant components (catalytic converters / particulate filter / SCR system / oxygen / NOx / particle sensors) are checked within the scope of the PTI...